Mackay School District No. 182

NONINSTRUCTIONAL OPERATIONS	8190

Emergencies Involving Transportation Vehicles

In the event of an accident or other emergency, the bus driver shall follow the emergency procedures developed by the Superintendent.  A copy of the emergency procedures shall be located in each bus.  To ensure the success of such emergency procedures, each bus driver shall conduct an emergency evacuation drill within the first six weeks of each school semester.  The District shall conduct such other drills and procedures as may be necessary.

To allow evacuation in the event of an emergency, items belonging to those riding the bus must be safely stowed and secured away from any aisle or emergency exit.  To ensure that aisles and emergency exits are kept clear at all times, the Superintendent may issue rules limiting the size or number of items riders may bring with them on the bus.

When evacuating the bus follow these procedures:

Evacuating through the front door

1.  Set park brake.

2.  Turn off engine.

3.  Put transmission in reverse (manual transmission) or neutral (automatic transmission).

4.  Turn on four-way hazard lamps (if operable).

5.  Driver should stand and face students.

6.  Test front service door to see if it working before making any announcements.

7.  Get the student’s attention – speak clearly and concisely.

8.  Evaluate the situation.  Determine if there are any injuries, and determine a safe waiting area to move the students to.

9.  Announce to the students to exit through the front, give the first aid kit, fire extinguisher and reflective triangles to responsible students.

10.  Check to make sure all students are out.

11.  Assist the injured to the best of your abilities.

12.  Notify proper authorities.

Evacuating through the rear and or side door

1.  Steps 1 through 4 in the front door evacuations need to be followed.

2.  Assign tow “helpers” to assist students.

3.  Instruct the helpers to “sit” on the floor at the emergency door and “scoot” out of the door onto the ground.

4.  One helper is positioned so that the emergency door will not swing against the students.  The other helper is positioned on the other side of door area.

5.  The helpers need to hold a hand open, palm upward and extended for the student to place his/her hand on it.  The other hand will support the upper part of the arm of the exiting student.

6.  Students should “squat” at the rear door to eliminate the possibility of hitting their heads.

7.  Have the student “drop” out of the bus.  Never have student “jump” out of the bus.

8.  Have them walk to the designated sage waiting area.

9.  Evacuate the bus, starting at back row and continue to the front.

10.  Give first aid kit, fire extinguisher, and emergency triangles to the last two students after they are out of the bus.

11.  Have the helpers assist you out of the rear of the bus.

12.  Assist the injured to the best of your ability.

13.  Account for all students.

14.  Notify proper authorities as soon as possible.

Reference: 	Federal Highway Safety Guideline 17

Policy History:

Adopted on: August 8, 2016

Revised on: December 11, 2023